# HRS Installation instructions

#### Installation

- Notice:Currently only supports Windows 7 and Windows 10 operating systems

- 1.1 Download the HRS installation package. Unzip it out there are two installation files HRS-xxx_install.exe and LAVFilters-0.68.1.exe. Double-click to install separately.

- 1.2 HRS icon appears on the desktop or start menu after successful installation，double-click the icon to run HRS.

#### Instructions

- 2.1Make sure your camera and computer are in the same network environment(under the same WiFi or Router).

- 2.2 Open HRS：

- 2.3 In normal circumstances, HRS will automatically search for cameras on the same LAN, then cameras will be displayed under the default grouping. If it doesn't appear, you can click the "Find Device" button in the lower left corner.

- 2.4 Double-click the camera and enter the camera's password to view live live stream.

- 2.5 Channels Switch，You can click the split screen button at the top right or select it from the pop-up menu in the playback window. Like shown below：

- 2.6 Sound,Record and Screenshots. In the case of video playback, you can click on the play window, the control bar will appear, and the corresponding button can be clicked.

- 2.7 View picture, Watch local recorded video and TF card video

- Click the album button in the lower left corner of the main interface.，open the album，like the follow:

- In the top left corner of the album, you can choose to view the screen shots photo,local video and TF card video, then double-click on the camera to view the image or video on the right panel. TF card video sample are shown like follow：

- 2.8 Motor Control

- Select the camera to play, click the “PTZ” button at the bottom of the main interface, you can control the camera’s motor ：